BUFR TABLES D

(Version 1 - originating center 65535)

| TABLE REFERENCE |  |  | TABLE REFERENCES |  |  | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- |
| F | X | Y |  |  |  |  |
|  |  |  |  |  |  | General information about radar product |
| 3 | 1 | 192 | 3 | 1 | 11 | Date of observation |
|  |  |  | 3 | 1 | 12 | Time of observation |
|  |  |  | 3 | 1 | 23 | Latitude/longitude of top left corner of the image |
|  |  |  | 3 | 1 | 23 | Latitude/longitude of top right corner of the image |
|  |  |  | 3 | 1 | 23 | Latitude/longitude of bottom right corner of the image |
|  |  |  | 3 | 1 | 23 | Latitude/longitude of bottom left corner of the image |
|  |  |  | 0 | 29 | 1 | Projection type |
|  |  |  | 0 | 5 | 2 | Latitude of radar |
|  |  |  | 0 | 5 | 2 | Latitude of radar |
|  |  |  | 0 | 5 | 33 | Pixel size on horizontal between columns |
|  |  |  | 0 | 6 | 33 | Pixel size on horizontal between rows |
|  |  |  | 0 | 30 | 21 | Number of pixels per row |
|  |  |  | 0 | 30 | 22 | Number of pixels per column |
|  |  |  |  |  |  | Heights of side view |
| 3 | 13 | 192 | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 10 | 7 | Height of side view |
|  |  |  |  |  |  | List of radars included in a composite |
| 3 | 21 | 13 | 1 | 4 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  | 0 | 1 | 1 | WMO block number |
|  |  |  | 0 | 1 | 2 | WMO station number |
|  |  |  | 0 | 31 | 31 | Data present indicator |
|  |  |  | 0 | 33 | 3 | Quality information |
|  |  |  |  |  |  | 4 bit per pixel radar images (top view) |
| 3 | 21 | 192 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  | 0 | 5 | 31 | Row number |
|  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  |  |  | 8 bit per pixel radar images (top view) |
| 3 | 21 | 193 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  | 0 | 5 | 31 | Row number |
|  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  |  |  | 4 bit per pixel radar image (north-south view) |
| 3 | 21 | 194 | 1 | 10 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  | 0 | 5 | 31 | Row number |
|  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  |  |  | 8 bit per pixel radar image (north-south view) |
| 3 | 21 | 195 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  | 0 | 5 | 31 | Row number |
|  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  |  |  | 4 bit per pixel radar image (east-west view) |
| 3 | 21 | 196 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  | 0 | 5 | 31 | Row number |
|  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 30 | 1 | Pixel value (4 bit) |
|  |  |  |  |  |  | 8 bit per pixel radar image (east-west view) |
| 3 | 21 | 197 | 1 | 10 | 0 | Delayed replication of 10 descriptors |
|  |  |  | 0 | 31 | 2 | Extended delayed descriptor replication factor |
|  |  |  | 0 | 5 | 31 | Row number |
|  |  |  | 1 | 7 | 0 | Delayed replication of 7 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 1 | 2 | 0 | Delayed replication of 2 descriptors |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 31 | 12 | Extended delayed descriptor and data repetition factor |
|  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  | 1 | 1 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Delayed descriptor replication factor |
|  |  |  | 0 | 30 | 2 | Pixel value (8 bit) |
|  |  |  |  |  |  | List of radars included in a composite |
| 3 | 21 | 250 | 1 | 4 | 0 | Delayed replication of 1 descriptor |
|  |  |  | 0 | 31 | 1 | Replication factor |
|  |  |  | 0 | 1 | 1 | WMO block number |
|  |  |  | 0 | 1 | 2 | WMO station number |
|  |  |  | 0 | 31 | 31 | Data present indicator |
|  |  |  | 0 | 33 | 3 | Quality information |